Delay estimation and compensation within Real-time Hybrid Simulation of a SDOF System on a BRB Specimen

Real-Time Hybrid Simulation (RTHS) is a novel approach conceived to evaluate dynamic responses of structures with parts of a structure physically tested and the remainder parts numerically modelled. One challenge of the method is to ensure real-time interaction of parts –substructures- via loading control, involving delay estimation, delay compensation and transfer system control. In RTHS, delay estimation is often a precondition of compensation; nonetheless, system delay may vary during testing. Consequently, it is sometimes necessary to measure delay online. Along these lines, we proposed an estimator with Taylor series based on a simplified physical system model, i.e. a pure delay multiplied by a gain reflecting amplitude errors of physical system control. The least-squares algorithm is introduced as well in order to render the estimator more practical yet effective.

In order to physically verify this method, Real-time Hybrid Simulation with a BRB specimen were carried out at the Mechanical and Structural Testing Center of the Harbin Institute of Technology and presented here. The emulated structure is a single degree of freedom structure, as shown in Fig.1. The specimen is a Buckling-Restrained Brace (BRB), which behaves linearly and nonlinearly in two tests, respectively.

Fig. 1 Schematic of the emulated structure in RTHS

A picture of the experimental substructure installed on the MTS servo-hydraulic actuator is illustrated in Fig. 2. This actuator is characterized by a dynamic loading capacity of 2500kN and a stroke of 300mm. In the tests, computation of the time-discretized equation of motion, delay estimation and delay compensation were performed in Calculation Editor of the control system of MTS servo-hydraulic actuator, i.e. Flex Test GT (MTS) (MTS System Corporation, 2001). The Calculation Editor provides an easy way to online process signals by programming. Fig.3 shows the MTS Station Manager together with the Calculation Editor.

Fig. 2 Actual test set-up for RTHS

Fig.3 The MTS Station Manager and Calculation Editor

The initial stiffness of the physical substructure was found to be 144×106 N/m from previous tests while the stiffness of the numerical substructure was chosen as half of that, i.e., kn=72×106 N/m. The mass of the system was chosen in such a way that the natural frequency of the overall elastic structure was 1Hz for all test cases. The damping of the numerical substructure was assumed to be zero. The Central Difference Method was used to solve the equation of motion with a time interval of 4.902ms. In view of the system delay of about 18ms, which was about three times the time interval value, the third-order polynomial extrapolation was carried out with structural responses at only even or odd steps. Therefore, the following expression was adopted for delay compensation:

and

As a result, structural responses at a larger time span were applied for displacement prediction; thus, the influence of higher-frequency components in structural displacement on predicted displacement can be suppressed.

Fig. 4 Time histories of estimated delay with the proposed method in linear RTHS

Fig. 5 Time histories of displacement responses with the proposed method in linear RTHS

# 1 Linear case

Fig. 4 presents the estimated delay of the proposed method with both the forgetting factor ρ=0.98 and ρ=0.99 compared with that provided by Darby’s method. Evidently, the two test results based on the proposed method provide similar delay histories and give estimated values consistent to Darby’s method after the first four seconds. In the first four seconds, the estimated values by the proposed method are greater than those provided by Darby’s method. However, as shown in Fig. 5, the measured displacement matches well the desired one after the first 0.5s; this implies that the delay is not over-compensated for in the beginning of the test. Note that in the first 0.5s, the system delay is not compensated for and hence, the displacement command and the desired displacement are identical to each other. Consequently, the estimated delay of Darby’s method is less than the actual delay. In fact, it is likely that the system delay is greater in this stage since the loading system has to switch from the static state to movement. In addition, the tested displacement provided in the simulation with ρ=0.98 matches the predicted displacement well.

# 2 Nonlinear case

In this section, the PGA is tuned to 0.122g. Fig. 6 shows the estimated delay histories provided by Darby’s method; they exhibit oscillations. The hysteretic relationship of the specimen corresponding to the test with Cp = 0.01 is shown in Fig. 7; it is characterized by a maximum displacement of about 4mm and a maximum restoring force of about 400kN, respectively.

Fig. 6 Time histories of the estimated delay with Darby's method in nonlinear RTHS

Fig. 7 Hysteretic response of the specimen

Fig. 8 plots the estimated delay histories in RTHS with the proposed estimation method. In order to investigate the repeatability of the testing method, each test was carried out twice. This figure illustrates response characteristics similar to those described above. However, the delay oscillation is smaller. In fact, as aforementioned, the proposed method measures the system delay with both the displacement command and displacement response; therefore, the delay could be measured off line. This is not possible with both Ahmadizadeh’s and Darby’s methods. The estimated delay histories with different parameters are plotted in Fig. 9. A careful reader can observe that the off line estimated-delay histories with ρ = 0.95 and ρ = 0.98 are almost identical to the online measured result in the second test with ρ = 0.98. This indicates that the smoother estimated delay cannot be attributed to a larger forgetting factor, and hence the delay in the nonlinear tests may not change greatly. Actually, the MTS loading facility has a loading capacity of 2500kN, about 6.3 times the maximum restoring force, and thus in the test, the nonlinearity of the actuator is not apparent. In addition, the tangent stiffness of the specimen did not change too much during the tests; therefore, the delay did not greatly vary during simulations. Moreover, Fig. 18 indicates that the parameter ρ = 0.98 is a good candidate for common tests; this implies that the parameter choice for the proposed method is easy and the overall method quite practical.

| Fig. 8 Time histories of the estimated delay with the proposed method in nonlinear RTHS | Fig. 9 Comparisons of online and offline estimated delays |
| --- | --- |

In addition, RTHS with the proposed method exhibits better relative displacement errors in the first two seconds. This can be ascribed to the proper-compensated delay in this time range; in the subsequent time range, two test results are similar, which may result from hysteretic damping. More detailed results can be found in Wang (2012).