Executive Summary:

This project reports the results of an experimental study conducted to demonstrate the feasibility and capability of magnetorheological (MR) dampers commanded by a decentralized control algorithm for seismic control of nonlinear civil structures considering soil-structure interaction (SSI). A two-story reinforced concrete (RC) frame resting in a laminar soil container is employed as the test specimen, and two MR dampers equipped in the ﬁrst story are used to mitigate the response of this frame subjected to various intensity seismic excitations. A hyperbolic tangent function is used to represent the hysteretic behavior of the MR damper and a decentralized control approach for commanding MR dampers is proposed and implemented in the shaking table tests. Only the response of the ﬁrst story is feedback for control command calculation of the MR dampers. The results indicate that the MR damper can effectively reduce the response of the soil-structure system, even when the soil-structure system presents complex nonlinear hysteretic behavior. The robustness of the proposed decentralized control algorithm is validated through the shaking table tests on the soil-structure system with large uncertainty. The most interesting ﬁndings in this project are that MR dampers not only mitigate the superstructure response, but also reduce the soil response, pile response and earth pressure on the pile foundation.

A two-story, single-bay, reinforced concrete (RC) frame is employed as the specimen in this study. Experimental investigations are performed in the Structure Laboratory at the Institute of Engineering Mechanics (IEM), China Earthquake Administration. The test structure used in this experiment as shown in ﬁgure 1 is a two-story reinforced concrete frame. A   laminar   container   aimed   at   simulating free   field boundary conditions of the soil deposit is shown in figures 1 and 2.  The pile foundation designed for this study is composed of an RC pile cap and four RC piles. The pile cap is designed to have sufficient rigidity, with 2.0 m × 2.0 m in plan and 0.15 m in thickness.  The mass of the pile cap is 1.5 ton.  The piles have the same cross section as the columns of the superstructure. Two MR dampers are incorporated into the first story of the structure in two bays. The load cells and linear variable differential transformers (LVDT) are installed to respectively measure the damper force and damper deformation, as shown in figures 1 and 2.    The box is comprised of 15 rectangular steel rings. Linear roller bearings are arranged between rings to permit relative movement in the longitudinal direction with minimal friction.

The drifts of the structure and control force under the high level input are shown in figure 8.   These results  show that  the  control  force  of  semi-active  control  is  quite  the same as that of passive-on control, resulting in a similar reduction  in structural  response,  as listed in table 2.   In the high  PGA  case,  the  reduction  achieved  by  the  semi-active control is better than other PGA cases on both displacement responses  and  acceleration  responses;  the drifts  of pile cap and superstructure are all reduced over 50% and all their accelerations   are  smaller  than  those  without  control;   and as shown in table 2, the uncontrolled structure undergoes remarkable damage.  Focusing attention on the magnitude of the control force, it can be observed that the maximum control force of passive-off control is only 200 N in contrast to 900 N for both semi-active control and passive-on control. Therefore, the passive-off control cannot achieve the high reduction as the passive-on control and semi-active control do.

Figure 1. Photo of the test set-up on the IEM shaking table and the detailed design results of the frame: (a) top view, (b) front view

Figure 2. Schematic diagram of the control test set-up.

Through shaking table tests of a two-story RC frame resting in a laminar soil container, the feasibility, capability and robustness of the MR damper commanded by the decentralized control methodology have been validated.  Nonlinear behavior of the test RC structures is treated as an unknown nonlinear term and the soil-structure interaction is treated as an additional external excitation in this control strategy.   For all selected PGA levels, the mitigation in response achieved by both semi- active control and passive-on control is quite similar in this experiment. In contrast, the control effectiveness of passive- off control is slight, especially at the low PGA and medium PGA cases.  In the high PGA case, the MR damper with semi-active control and passive-on control can well prevent the structure from severe damage and the reduction achieved by the MR dampers is remarkable. In addition, both superstructure response and soil response can be mitigated by the MR dampers. The mitigation in soil response can decrease the earth pressure, i.e. the input to the superstructure.